Task 3: Circuit Design & Power Budget

# 3a. Schematic and wiring diagram

## Why each choice

Why servos must not run from the ESP32's 5 V pin. That pin is fed from USB or Vin through a small on-board regulator and a protection fuse, good for a few hundred milliamps. One moving MG996R draws about 0.5 A and up to 2.5 A when it stalls, and five servos can pull several amps together. The voltage would sag, the ESP32 would reset (brown-out), and the servo noise would corrupt its logic. A servo also gives more torque at 6 V than at 5 V (11 against 9.4 kg.cm), and a dedicated buck rated for the load solves both.

What the motor driver does. The ESP32 pins can only supply a few milliamps at 3.3 V, so they cannot drive a 12 V motor. The L298N is two H-bridges: for each motor, IN1/IN2 choose which pair of transistors conducts (so the current, and the rotation, goes forward, backward, or brakes), and the PWM on ENA/ENB switches the bridge on and off to set the average voltage and therefore the speed. It also contains the diodes that absorb the motor's voltage spikes, protecting the ESP32.

Wire gauge for the high-current paths. Silicone wire, which stays flexible and tolerates heat. I chose it so the voltage drop is below about 1 % and the wire is never the weakest link next to a 15 A fuse.

| Path | Design current | Gauge | Why |
| --- | --- | --- | --- |
| Battery to switch to fuse to bus (trunk) | up to 8.8 A, fuse 15 A | 14 AWG (2.1 mm2) | Carries everything; resistance about 8 mOhm per metre, so 0.6 m at 8.8 A drops only about 0.05 V |
| Bus to L298N Vs | 4 A running, higher in surges | 16 AWG | Short run; 16 AWG is comfortable well above 10 A |
| Bus to 6 V buck input | 4.5 A worst case | 16 AWG | Input current is lower than the 6.7 A output because 11 V is converted down to 6 V |
| 6 V buck output to servo rail | 6.7 A design worst case | 16 AWG trunk, 20 AWG tails to servos | Short and thick keeps the rail stiff; each servo has its own pair |
| Driver output to motors | 1 A running, 4 A stall | 20 AWG | Matches the motor tab size and current |

Wireless and fail-safe (design intent). The ESP32's own Wi-Fi/Bluetooth is the receiver. It accepts commands only from my controller (paired by its address) and ignores everyone else. A 300 ms watchdog runs in firmware: if no valid packet arrives, the motors are switched off (ENA = ENB = 0), the servo pulses stay at their last value (arm and gripper hold position), the status LED on GPIO2 shows the lost link, and the robot stays disabled until a new valid command arrives.

# 3b. Electronics BOM, power budget and battery

Budget: INR 12,000 including shipping and about 10 % contingency. Items INR 10,400 + shipping INR 350 = INR 10,750; 10 % contingency = INR 1,075; total INR 11,825, which leaves INR 175 spare. Prices marked *listing* were read from a supplier page; the rest are my estimates for budgeting and must be re-checked at checkout (the motor model in particular).

| # | Item (exact model / type) | Qty | Unit INR | Total INR |
| --- | --- | --- | --- | --- |
| 1 | ESP32 DevKit V1 (ESP32-WROOM-32, 30-pin) | 1 | 450 | 450 |
| 2 | L298N dual H-bridge module with heatsink | 1 | 160 | 160 |
| 3 | 12 V 100 rpm metal-gear DC motor (Johnson-type, 37 mm, ~18 kg.cm stall) | 4 | 520 | 2080 |
| 4 | MG996R metal-gear servo (11 kg.cm @ 6 V) | 4 | 340 | 1360 |
| 5 | MG90S micro servo (gripper) | 1 | 150 | 150 |
| 6 | DC-DC buck 20 A adjustable (set 6.0 V) - servo rail | 1 | 750 | 750 |
| 7 | LM2596 buck 3 A module (set 5.0 V) - logic rail | 1 | 130 | 130 |
| 8 | 3S LiPo 2200 mAh 25C, XT60 | 1 | 1900 | 1900 |
| 9 | LiPo balance charger (iMAX B6-class) + 12 V adapter | 1 | 1800 | 1800 |
| 10 | Kill switch: 30 A DC-rated rocker/latching, panel mount, red | 1 | 180 | 180 |
| 11 | Inline blade-fuse holder (14 AWG) + 3x 15 A ATM fuses | 1 | 120 | 120 |
| 12 | XT60 connector pairs (battery lead + spare) | 2 | 60 | 120 |
| 13 | 14 AWG silicone wire, red + black, 2 m each | 1 | 250 | 250 |
| 14 | 16/18/20 AWG silicone wire assortment (servo, motor, logic) | 1 | 200 | 200 |
| 15 | Capacitors: 2x1000 uF/25 V, 2x1000 uF/16 V, 6x470 uF/25 V, 10x100 nF, 2x10 uF | 1 | 180 | 180 |
| 16 | WAGO 221 lever connectors / screw terminal blocks (power & GND star) | 1 | 220 | 220 |
| 17 | Dupont/JST connectors, 3-pin servo extensions, heat-shrink, cable ties | 1 | 200 | 200 |
| 18 | Heatsink + thermal paste for L298N | 1 | 60 | 60 |
| 19 | Battery strap + hook-and-loop + foam | 1 | 90 | 90 |
|  | Items subtotal |  |  | 10400 |
|  | Shipping (estimate) |  |  | 350 |
|  | 10 % contingency |  |  | 1075 |
|  | TOTAL |  |  | 11825 |

## Power budget by rail

Idle = powered but not working. Typical = what the part draws in normal use. Peak = stall, the worst case. Motor and servo reference values are from the task's Appendix A; I note where my parts' datasheets may differ.

| Rail / component | Qty | Idle each (A) | Typical each (A) | Peak / stall each (A) | Idle total | Peak total |
| --- | --- | --- | --- | --- | --- | --- |
| 11.1 V motor supply (via L298N) |  |  |  |  |  |  |
| 12 V 100 rpm gearmotor | 4 | 0 (stopped; 0.25 no-load) | 1.0 | 4.0 | 0.00 | 16.0 |
| 6.0 V servo rail |  |  |  |  |  |  |
| MG996R servo | 4 | 0.010 | 0.50 (moving) | 2.5 | 0.040 | 10.0 |
| MG90S gripper servo | 1 | 0.005 (assumed) | 0.25 (assumed) | 0.7 | 0.005 | 0.7 |
| 6 V rail total |  |  |  |  | 0.045 | 10.7 |
| 5.0 V logic rail |  |  |  |  |  |  |
| ESP32 DevKit | 1 | 0.08 | 0.20 | 0.35 | 0.08 | 0.35 |
| L298N logic (Vss) + indicator LED | 1 | 0.05 (assumed) | 0.05 | 0.05 | 0.05 | 0.05 |
| 5 V rail total |  |  |  |  | 0.13 | 0.40 |

Worst-case design load on the 6 V rail = 6.7 A: shoulder and elbow stalled together (2 x 2.5 A), base and wrist moving (2 x 0.5 A), gripper stalled (0.7 A). All five stalled at once would be 10.7 A, which cannot happen because the arm joints do not all fight a load at the same moment, but I size for it anyway.

| Converter | Worst-case load | x 1.5 margin | Module chosen | Margin achieved |
| --- | --- | --- | --- | --- |
| 6 V servo buck | 6.7 A (design); 10.7 A (all stalled) | 10.1 A; 16.0 A | 20 A-class adjustable buck | 3.0x the design load; 1.9x even if all stall |
| 5 V logic buck | 0.40 A (ESP32 radio peak + L298N logic) | 0.6 A (the task sheet suggests at least 2 A) | LM2596 3 A module | 7.5x |

Cheap 20 A modules are usually rated at full current only with a big heatsink and airflow. Even if I count only 10 A of real continuous rating, that is still 1.5x the design worst case, and I will fit a heatsink.

What the battery sees. Buck inputs are calculated as P(out) / efficiency / battery voltage (efficiency 0.90 for the 6 V buck, 0.85 for the 5 V buck; battery voltage 11.1 V, or 9.9 V for the worst case).

| Condition | Motors | 6 V buck input | 5 V buck input | Total from battery |
| --- | --- | --- | --- | --- |
| Idle (all powered, nothing moving) | 0 A | 0.04 A | 0.07 A | 0.11 A |
| Typical, driving and moving the arm | 4.0 A | 0.90 A (1.5 A at 6 V) | 0.13 A | 5.03 A |
| Run average (driving 60 % of the time) | 2.4 A | 0.90 A | 0.13 A | 3.43 A |
| Design worst case (L298N limit 2 A per channel) | 4.0 A | 4.51 A (at 9.9 V) | 0.24 A | 8.75 A |
| Everything stalled (not survivable; fuse blows) | 16 A | 7.21 A | 0.24 A | 23.4 A |

The 15 A fuse is 1.7x the design worst case (8.75 A) and well below the 14 AWG wire rating, so it protects the wiring in a short circuit without nuisance blowing.

## Battery choice and runtime

Battery: 3S LiPo, 2200 mAh, 11.1 V, 25C (about 190 g). Assumptions: typical load means the run-average above (driving about 60 % of the time with all four motors at 1.0 A, arm and gripper working, logic on), and I assume only 80 % of the capacity is usable (stopping at 3.3 V per cell = 9.9 V, so the battery is never over-discharged).

| Quantity | Value |
| --- | --- |
| Usable capacity | 2.2 Ah x 0.80 = 1.76 Ah |
| Average current over a run | 3.43 A |
| One 7-minute run uses | 3.43 A x 7/60 h = 0.40 Ah (18 % of the nominal capacity) |
| 7 min run + 10 min of testing | 0.97 Ah (44 % of nominal), within the 1.76 Ah usable |
| Continuous runtime at the average load | 1.76 / 3.43 = 31 min, about four full runs per charge |
| Runtime at the design worst case | 1.76 / 8.75 = 12 min, still longer than one 420 s run |
| Discharge rate check | Capacity x C-rating = 2.2 x 25 = 55 A available, against 8.7 A worst case (about 4C): lots of headroom |

## Chemistry comparison: LiPo vs Li-ion 18650

| Property | 3S LiPo 2200 mAh 25C (my pick) | 3S1P Li-ion 18650 (2500 mAh, 20 A cells) with BMS |
| --- | --- | --- |
| Energy | about 24 Wh | about 27 Wh (cells are 2500 mAh at 10.8 V) |
| Energy density (pack) | about 130 Wh/kg (190 g) | about 160 Wh/kg (about 165 g with holder and BMS) |
| Continuous current | 55 A (25C) | 20 A (cell limit; cheap BMS often cuts at 10 to 20 A) |
| Voltage sag under load | low (stiff pack, a few tens of mOhm) | higher (about 2x internal resistance), dips when servos start |
| Weight | about 190 g | about 165 g |
| Cost (India, indicative) | about INR 1,900 | about INR 1,300 to 1,700 (cells + holder + BMS) |
| Shape / mounting | flat pouch, low in the chassis, XT60 plug | cylinders in a holder; needs a BMS and spot-welded or holder contacts |
| Safety | soft pouch: needs balance charging, a LiPo bag, no puncture | metal can is more rugged; BMS gives protection |

My pick: LiPo. I only need about 1 Ah per session, so Li-ion's extra Wh does not matter, while the LiPo's far higher current rating and low voltage sag matter a lot when two MG996R stall in the same instant (a sagging bus is what resets controllers). A Li-ion 3S1P is a perfectly good alternative and a bit lighter; I would take it if the arena rules or safety review preferred cylindrical cells. Values are typical for these cell classes; I will check the datasheet of the exact pack I buy.

## Compliance checks

24 V limit: 3S fully charged = 3 x 4.2 V = 12.6 V (the highest voltage anywhere), compliant. A 6S pack would be 25.2 V and fail the rule. There is no boost converter, so no node exceeds the battery voltage.

Kill switch current rating: the switch sits in the battery positive lead before everything, so it shuts down the bucks, driver, ESP32 and servos together. Design current is 8.8 A and the fuse is 15 A, so I use a 30 A DC-rated switch (at least 2x the fuse, so the switch is never the weak link). I check that the rating is for DC at 12-24 V, because AC ratings are higher than DC ratings. It is mounted on the outside, red, easy to reach.

Battery cannot detach if the bot tips: the pack sits low on the chassis floor in a plywood or printed cradle with a front and rear stop, held by two hook-and-loop straps through slots in the chassis. The XT60 plug is strain-relieved with a small service loop and a cable tie to the chassis, so a pulled cable cannot unplug it. I will test with a 45 degree tilt and an upside-down shake before the run.

# 3c. Datasheet check: L298N motor driver

I chose the L298N because it is the driver on my schematic and on the reference sheet. These are the specs that shaped the design.

Spec 1: 2 A continuous per channel (about 3 A for very short peaks). The task sheet gives a motor stall current of 4 A, which is already twice the driver rating with one motor per channel. My circuit puts two motors on each channel: running at 1 A each they load a channel with 2 A (100 % of the rating), and a stall would demand 8 A (4x the rating). This forced me to add protection: a heatsink, soft-start ramps in the PWM, a 0.5 ohm sense resistor on the SENSE pins to read the current and cut the PWM when a channel exceeds about 2.5 A for 50 ms, and the 15 A fuse for the wiring. The BOM also keeps INR 900 of contingency free for an upgrade (below).

Spec 2: voltage drop of the bridge, about 2 to 3 V total (the task sheet says about 2 V, the datasheet curves are somewhat higher at 1 to 2 A). The motors therefore see only about 9 V from a 11.1 V battery, which is why I chose a 3S pack. A 2S pack (7.4 V) would leave only about 5 V and about 40 % of the speed. The drop also heats the chip: about 2 A x 2.5 V x 2 channels = 10 W, so I fit a heatsink.

Spec 3 (checked, no change needed): the logic inputs switch at about 2.3 V, so the ESP32's 3.3 V outputs drive them directly without a level shifter.

A condition that would damage the part: sustained current above about 2 A per channel, or a stalled motor, overheats the silicon. It first goes into thermal shutdown and, if it keeps happening, it is destroyed. A second one is putting more than 7 V on the logic supply Vss.

How my design avoids it: the driver is never allowed to see a stall: PWM is ramped up over about 100 ms; the SENSE shunt cuts the channel when the current stays above the limit; the heatsink keeps the chip cool; and Vss is fed from the 5 V buck with the module's regulator jumper removed, so the logic supply is always 5 V.

# 3d. Troubleshooting: (a) the controller resets when the shoulder servo starts moving

### Probable root causes

| # | Cause | Why it makes the ESP32 reset |
| --- | --- | --- |
| 1 | Supply dip (brown-out) on the 5 V rail | The shoulder servo's start-up and stall current (up to 2.5 A) makes the 6 V buck draw a spike from the 11.1 V bus. The bus dips, the 5 V buck input follows, and Vin falls low enough that the ESP32's brown-out detector fires |
| 2 | Shared or thin ground return | If servo return current and the ESP32 ground use the same wire or breadboard rail, ground rises by a few hundred millivolts and the controller's logic reference moves |
| 3 | Not enough local capacitance | Without 470 to 1000 uF at the servo header and 1000 uF at the battery input, nothing stores energy for the spike |
| 4 | 6 V buck too weak or in current-limit | A cheap module that hits its limit collapses its output or restarts, which also drags the bus |
| 5 | Loose connector or high-resistance joint | A Dupont plug on a power line adds resistance and sags under 2 A |

First diagnostic check. Print the ESP32 reset reason at start-up (esp_reset_reason). If it reads brown-out, the cause is power, not code. At the same time, measure the voltage at the ESP32 Vin pin and at the shoulder servo header with a multimeter in min-hold mode (or a scope in single-shot trigger) while commanding the shoulder to move from rest. A quick isolating test: power the ESP32 from USB only, with the servo still on the buck; if the resets stop, the problem is supply coupling.

The fix, in order. Add 1000 to 2200 uF at the shoulder header and 1000 uF at the battery input; give the servo rail and the ESP32 separate ground wires back to the star point; use the 16 AWG trunk for the servo rail; and slew-limit the shoulder command so it ramps instead of jumping (this cuts the starting current peak).

How I confirm the fix. (1) The reset-reason log shows no brown-outs, and an uptime counter keeps running through 100 shoulder sweeps carrying a load. (2) The scope or min-hold reading shows Vin staying above about 4.7 V and the 6 V rail above about 5.4 V at the worst moment. (3) A full 7-minute dummy run (drive plus arm plus gripper) completes with no reset.